SPECIFICATIONS: HYDRA-MATIC 6T70 / 6T75

|  | 2007 6T70 ( MH2 ) | 2007 6T75 MH6 MY9 |
| --- | --- | --- |
| Type: | Six speed front-wheel-drive, electronically controlled, automatic overdrive transaxle with an electronically controlled torque converter clutch. | Six speed front-wheel-drive & AWD, electronically controlled, automatic overdrive transaxle with an electronically controlled torque converter clutch. |
| Engine range: | 4.0L | 4.6L |
| Maximum engine torque: | 280 lb-ft (380 Nm) | 301 lb-ft (406 Nm) |
| Maximum gearbox torque: | 462 lb-ft (515Nm) | 463 lb-ft (515Nm) |
| Gear ratios: |  |  |
| First | 4.48 | 4.48 |
| Second | 2.87 | 2.87 |
| Third: | 1.88 | 1.88 |
| Fourth: | 1.47 | 1.47 |
| Fifth | 1.00 | 1.00 |
| Sixth | 0.74 | 0.74 |
| Reverse: | 2.88 | 2.88 |
| Final drive ratio: | 2.77 | 3.16:1 |
| Maximum shift speed: | 1-2: 6800 rpm | 1-2: 6800 rpm |
|  | 2-3: 6800 rpm | 2-3: 6800 rpm |
|  | 3-4: 6800 rpm | 3-4: 6800 rpm |
| Maximum validated gross vehicle weight: | 4850 lbs | 6384 lbs |
| Shifter positions: | P, R, N, M | P, R, N, M |
| Case material: | die cast aluminum | die cast aluminum |
| Shift pattern: | 4 VBS Solenoids | 4 VBS Solenoids |
| Shift quality: | Variable Bleed solenoid | Variable Bleed solenoid |
| Torque converter clutch: | 258 mm | 258 mm |
| Converter size: | 246 mm (reference) (diameter of torque converter turbine) | 246 mm (reference) (diameter of torque converter turbine) |
| Fluid type: | DEXRON® VI | DEXRON® VI |
| Transmission weight: | wet: 102 kg | wet: 104 kg |
| Fluid capacity (approximate): | 9.0 L | 9.0 L |
| Bottom pan removal: | NA | NA |
| Pressure taps available: | line pressure | line pressure |
| Transfer design: | three-axis design | three-axis design |
| Assembly sites: | Warren , Mich. | Warren , Mich. |
| Applications: | Saturn Aura | Saturn Outlook |
|  | Pontiac G6 | GMC Acadia |
|  |  | Buick Enclave |