SPECIFICATIONS: HYDRA-MATIC 6L50

|  | 2007 Hydra-Matic 6L50 (MYB) |
| --- | --- |
| Type: | Six speed RWD / AWD, electronically controlled automatic overdrive transmission with torque converter clutch. Clutch-to-clutch architecture, with integral Electro / Hydraulic Controls Module |
| Maximum engine power: | 315 bhp (235 kW) |
| Maximum engine torque: | 332 lb-ft (450 Nm) |
| Maximum gearbox torque: | 480 lb-ft (651Nm) |
| Gear ratios: | MYB |
| First: | 4.06 |
| Second: | 2.37 |
| Third: | 1.55 |
| Fourth: | 1.16 |
| Fifth: | 0.85 |
| Sixth: | 0.67 |
| Reverse: | -3.2 |
| Maximum shift speed | 7000 rpm |
| Maximum Validated Weights: ( Target ) | GVW: 6613 lb (3000 kg), GCVW: 12505 lb (5672kg) |
| 7-position quadrant: | P, R, N, D, X, X, X ( X = available calibratable range position ) |
| Case description: | 3-piece ( Bell, main, extension) |
| Case material: | die cast aluminum |
| Shift pattern: | (2) Three-way on/off solenoids |
| Shift quality: | Five variable bleed solenoid |
| Torque converter clutch: | Variable Bleed Solenoid ECCC |
| Converter size: | 258, 280mm (reference) |
| Fluid type: | DEXRON® VI |
| Fluid capacity: w/258mm converter | TBA |
| Transmission weight: w/258mm converter | wet: 85-90 kg (187-198 lb) estimated |
| Pressure taps available: | line pressure |
| Assembly sites: | GMPT Ypsilanti, Mich. |
| Applications: | Cadillac STS |
|  | Cadillac SRX |
| Available control features: | Multiple shift patterns (selectable or adaptive) |
|  | Driver Shift Control (Tap Up / Tap Down) |
|  | Enhanced Performance Algorithm Shifting (PAS II) |
|  | Next-generation tow / haul mode |
|  | Engine torque management on all shifts |
|  | Altitude and Temperature Compensation |
|  | Adaptive shift time |
|  | Neutral idle |
|  | Reverse lockout |
|  | Automatic grade braking |
| Additional features: | GM Oil Life System |
|  | OBDII / EOBD |
|  | Integral Electro/Hydraulic Controls Module -- TEHCM |
|  | Control Interface Protocol - GMLAN |