SPECIFICATIONS: HYDRA-MATIC 6L80

|  | 2007 Hydra-Matic 6L80 (MYC) |
| --- | --- |
| Type: | Six-speed RWD / AWD, electronically controlled automatic overdrive transmission with torque converter clutch. Clutch-to-clutch architecture, with integral Electro / Hydraulic Controls Module |
| Maximum engine power: | 413 bhp (308 kW) |
| Maximum engine torque: | 430 lb-ft (583 Nm) |
| Maximum gearbox torque: | 664 lb-ft (900 Nm) |
| Gear ratios: | MYC |
| First: | 4.03 |
| Second: | 2.36 |
| Third: | 1.53 |
| Fourth: | 1.15 |
| Fifth: | 0.85 |
| Sixth: | 0.67 |
| Reverse: | -3.06 |
| Maximum shift speed | 6500 rpm |
| Maximum validated weights:(target ) | GVW: 8600 lb (3901 kg), GCVW: 14000 lb (6350 kg) |
| 7-position quadrant: | P, R, N, D, X, X, X (X = available calibratable range position ) |
| Case description: | Three-piece ( Bell, main, extension) |
| Case material: | die cast aluminum |
| Shift pattern: | (2) Three-way on / off solenoids |
| Shift quality: | Five variable bleed solenoids |
| Torque converter clutch: | Variable Bleed Solenoid ECCC |
| Converter size: | 300 mm ( reference ) |
| Fluid type: | DEXRON® VI |
| Fluid capacity: w/ 258 & 300mm converter | 258 mm: 9.7 L (8.22 kg), 300 mm: 11.9 L (10.1 kg) |
| Transmission weight: w/ 258 & 300mm converter | 258 mm: Wet: 94-96 kg (207-211 lb) |
|  | 300 mm: Wet: 102-104 kg (225-229 lb) |
| Pressure taps available: | line pressure |
| Assembly site: | GMPT Ypsilanti, Mich. |
|  | GMPT Toledo, Mich. (production startup 2008) |
| Applications: | Chevrolet Corvette |
|  | Cadillac XLR |
|  | Cadillac XLR-V |
|  | Cadillac STS-V |
|  | GMC Yukon Denali |
|  | GMC Yukon Denali XL |
|  | Cadillac Escalade |
|  | Cadillac Escalade ESV |
|  | Cadillac Escalade EXT |
| Available Control Features: | Multiple shift patterns (selectable or adaptive) |
|  | Driver Shift Control (Tap Up / Tap Down) |
|  | Enhanced Performance Algorithm Shifting (PAS II) |
|  | Next-generation tow / haul mode |
|  | Engine torque management on all shifts |
|  | Altitude and temperature compensation |
|  | Adaptive shift time |
|  | Neutral idle |
|  | Reverse lockout |
|  | Automatic grade braking |
| Additional Features: | GM Oil Life System |
|  | OBDII / EOBD |
|  | Integral Electro/Hydraulic Controls Module – TEHCM |
|  | Control Interface Protocol - GMLAN |